/*

SMS Your Home

This is a code to switch home appliance ON/OFF through sending SMS

This example shows an application on 1Sheeld's SMS shield.

*/

#define CUSTOM_SETTINGS

#define INCLUDE_SMS_SHIELD

#define INCLUDE_TERMINAL_SHIELD

/* Include 1Sheeld library. */

#include <OneSheeld.h>

/*

A name for the LED on pin 12.

A name for the Lamp on pin 13.

*/

int WledPin = 12;

int Lled1Pin = 13;

int Lled2Pin = 11;

int FanPin = 10;

void setup()

{

/* Start communication. */

OneSheeld.begin();

/* Set the LED pin as output.

set pin 12 as output

*/

pinMode(WledPin, OUTPUT);

pinMode(Lled1Pin, OUTPUT);

pinMode(Lled2Pin, OUTPUT);

pinMode(FanPin, OUTPUT);

}

void loop ()

{

SMS.setOnSmsReceive(&readSMS);

}

void readSMS (String phoneNumber , String messageBody)

{

//confirm sender is a specific numer//

String strMobileNumber = "+966598015002";

/* Print out the phone Number and message on Terminal shield. */

Terminal.println(phoneNumber);

Terminal.println(messageBody);

if (phoneNumber == strMobileNumber)

{

//can detect upper and lower case//

messageBody.toLowerCase();

//turn ON white led on pin 12 if "white on is recieved"//

if (messageBody == "white on")

{

digitalWrite(WledPin, HIGH);

}

//turn OFF white led on pin 12 if "white off is recieved"//

else if (messageBody == "white off")

{

digitalWrite(WledPin, LOW);

}

//turn ON lamp on pin 13 if "lamp1 on is recieved"//

else if (messageBody == "lamp1 on")

{

digitalWrite(Lled1Pin, HIGH);

}

//turn OFF lamp on pin 13 if "lamp1 off is recieved"//

else if (messageBody == "lamp1 off")

{

digitalWrite(Lled1Pin, LOW);

}

//turn ON lamp on pin 11 if "lamp2 on is recieved"//

else if (messageBody == "lamp2 on")

{

digitalWrite(Lled2Pin, HIGH);

}

//turn OFF lamp on pin 11 if "lamp2 off is recieved"//

else if (messageBody == "lamp2 off")

{

digitalWrite(Lled2Pin, LOW);

}

//turn ON lamp on pin 10 if "fan on is recieved"//

else if (messageBody == "fan on")

{

digitalWrite(FanPin, HIGH);

}

//turn OFF lamp on pin 10 if "fan off is recieved"//

else if (messageBody == "fan off")

{

digitalWrite(FanPin, LOW);

}

}

}